[기획서] 뇌졸중 환자를 위한 맞춤형 재활 게이미피케이션 시스템

프로젝트명: [추후 지정]

팀명: [추후 지정]

팀원: 이광은, 신영환, 연호준, 나예진[예정], 김지원[예정]

작성일: 2026. 01. 27.

1. 프로젝트 개요 (Overview)

- 주제: 저가형 IoT 센서와 게임 엔진(Unreal Engine)을 연동한 뇌졸중 상지(팔/손) 재활 훈련 시스템

- 목표: 대학생 창의 경연대회 출품 및 수상, 실제 환자가 사용 가능한 프로토타입 개발

- 핵심 가치: "지루하고 외로운 재활을 즐겁고 능동적인 경험으로 전환한다."

2. 기획 배경 및 시장 분석 (Problem & Solution)

2.1. 문제점 (Pain Point)

- 비용 장벽: 기존 병원용 재활 로봇은 수천만 원대로 일반 가정에서 구비가 불가능함.

- 낮은 지속성: 가정용 소형 기기는 단순 반복 운동 위주라 환자의 흥미 유발에 실패하여 중도 포기율이 높음.

- 개별화 부족: 환자마다 마비 정도(ROM: 가동 범위)가 다르지만, 기성품은 이를 세밀하게 반영하지 못함.

2.2. 해결방안 (Our Solution)

- 접근성 (Accessibility): 아두이노/ESP32 등 저가형 센서를 활용하여 하드웨어 비용을 획기적으로 절감.

- 몰입감 (Immersion): 언리얼 엔진의 고품질 그래픽을 활용한 '일상생활 시뮬레이션(ADL)' 게임 콘텐츠 제공.

- 기술적 차별점 (Tech Edge): 환자의 잔존 능력에 맞춘 '자동 보정(Calibration) 시스템' 탑재.

3. 주요 기능 및 시스템 구성 (Core Features)

A. 하드웨어 (Custom Controller)

- 구성: [예: 3D 프린팅된 핸들 + IMU(자이로) 센서 + 압력 센서]

- 기능: 환자의 손목 회전, 팔 들어 올리기, 악력 등의 물리적 데이터를 실시간 수집하여 PC로 전송.

B. 소프트웨어 (Game Client)

- 엔진: Unreal Engine 5

- 콘텐츠 구성 (Two-Track Strategy):

  - 기능성 훈련: 단순 수치 시각화 및 목표 달성 (예: 그래프 채우기, 게이지 바 올리기).

  - 일상생활 훈련(ADL): 가상 공간에서 문 열기, 컵 옮기기, 전등 끄기 등 실제 생활밀착형 동작 수행.

C. 미들웨어 (Data Processing) [핵심 경쟁력]

- 노이즈 필터링: 센서의 미세한 떨림 값을 보정하여 부드러운 움직임 구현 (이동평균 필터 등 적용).

- 개별화 (Calibration): 게임 시작 전 환자의 '최소/최대 가동 범위'를 측정 및 저장. 환자가 할 수 있는 최선의 움직임을 게임 내 100% 동작으로 매핑.

4. 개발 프로세스 및 로드맵 (Roadmap)

효율적인 협업과 리스크 관리를 위해 하드웨어(HW)와 소프트웨어(SW) 병렬 개발 후 통합하는 방식 채택.

| 단계 | 기간 | 하드웨어(HW) 팀 | 소프트웨어(SW) 팀 | 비고 |
| --- | --- | --- | --- | --- |
| Step 0 | 1주차 | 통신 프로토콜 정의 (패킷 구조: Header, Data, End 확정) | 기획 구체화 (게임 룰, 필요 데이터, 레벨 디자인 확정) | 가장 중요 |
| Step 1 | 2~3주차 | 센서 선정 및 회로 구성 시리얼 데이터 송출 테스트 | 더미(키보드/마우스) 입력 기반 게임 핵심 로직 구현 | 개별 개발 집중 |
| Step 2 | 4주차 | 하우징 제작 (3D 모델링) 착용감 개선 및 프로토타입 조립 | UI/UX 디자인 보정(Calibration) UI 및 로직 구현 | 중간 점검 |
| Step 3 | 5주차 | [통합] HW-SW 물리적 연동 입력 지연(Latency) 최적화 | [통합] 데이터 파싱 로직 적용 이펙트 및 사운드 폴리싱 | 최종 빌드 |

5. 기술 스택 (Tech Stack)

- Game Engine: Unreal Engine 5 (Blueprint & C++)

- MCU (Microcontroller): Arduino Uno 또는 ESP32 [신영환 보유중]

- Sensor: [MPU6050 (자이로/가속도) / 압력 센서 등 선정된 센서 기입]

- Communication: Serial Communication (UART) via UE4Duino Plugin

- Collaboration: GitHub

6. 팀원 역할 분담 (R&R)

- 팀장/PM: 전체 일정 관리, 기획서 고도화, 발표 자료(PPT) 및 시연 영상 준비

- HW 개발: 회로 설계 및 납땜, 센서 데이터 전처리 코딩, 하우징(3D) 제작

- SW 개발 (시스템): 언리얼-아두이노 시리얼 통신 연동, 데이터 보정 알고리즘 구현

- SW 개발 (콘텐츠): 게임 레벨 디자인, UI/UX 제작, 에셋 최적화

7. 기대 효과 (Expected Outcome)

- 사회적 측면: 고가의 재활 치료비 부담 경감 및 즐거운 재활 문화를 통한 삶의 질 향상.

- 기술적 측면: 하드웨어(IoT)와 게임 엔진 간의 실시간 양방향 상호작용 기술 확보.

- 대회 경쟁력: 단순한 게임이 아닌, '사회적 약자를 위한 따뜻한 기술'과 '실현 가능한 프로토타입'임을 강조하여 심사위원 설득.

[Project Proposal] Personalized Rehabilitation Gamification System for Stroke Patients

Project Name: [TBD]

Team Name: [TBD]

Team Member: Dave Lee, Younghwan Shin, Hojun Yeon, Yejin Na[ND], Noel Kim[ND]

Date: January 27, 2026

1. Project Overview

- Topic: Upper limb (arm/hand) rehabilitation training system using low-cost IoT sensors and a game engine (Unreal Engine).

- Goal: To participate in and win the University Creativity Competition; to develop a working prototype usable by actual patients.

- Core Value: "Transforming boring and lonely rehabilitation into a fun and active experience."

2. Background & Market Analysis

2.1. Pain Points

- Cost Barrier: Professional hospital rehabilitation robots cost tens of thousands of dollars, making them impossible for general households to own.

- Low Adherence: Existing home-care devices focus on simple repetitive movements, leading to a failure in sustaining patient interest and a high dropout rate.

- Lack of Personalization: Every patient has a different Range of Motion (ROM) due to paralysis, but off-the-shelf products fail to reflect these individual differences precisely.

2.2. Our Solution

- Accessibility: Drastically reduce hardware costs by utilizing low-cost sensors like Arduino and ESP32.

- Immersion: Provide "Activities of Daily Living (ADL) Simulation" game content utilizing the high-quality graphics of Unreal Engine 5.

- Tech Edge: Implement an "Auto-Calibration System" that tailors the control sensitivity to the patient's residual physical ability.

3. Core Features & System Architecture

A. Hardware (Custom Controller)

- Composition: [e.g., 3D Printed Handle + IMU (Gyro) Sensor + Pressure Sensor]

- Function: Real-time collection and transmission of physical data such as wrist rotation, arm lifting, and grip strength to the PC.

B. Software (Game Client)

- Engine: Unreal Engine 5

- Content Strategy (Two-Track):

  - Functional Training: Simple visualization of numerical data and goal achievement (e.g., Filling graphs, raising gauge bars).

  - ADL Training: Performing real-life tasks in a virtual space, such as opening doors, moving cups, or turning off lights.

C. Middleware (Data Processing) [Key Competitiveness]

- Noise Filtering: Smoothing out sensor jitter to ensure fluid movement (utilizing Moving Average Filters, etc.).

- Calibration (Personalization): Measuring and saving the patient's "Minimum/Maximum Range of Motion" before the game starts. Mapping the patient's best possible movement to 100% action within the game.

4. Development Process & Roadmap

Adopting a parallel development strategy for Hardware (HW) and Software (SW), followed by integration to ensure efficient collaboration and risk management.

| Step | Duration | Hardware (HW) Team | Software (SW) Team | Note |
| --- | --- | --- | --- | --- |
| Step 0 | Week 1 | Define Communication Protocol (Packet Structure: Header, Data, End) | Refine Planning (Game Rules, Required Data, Level Design) | Critical |
| Step 1 | Week 2~3 | Sensor selection & Circuit design Serial data transmission test | Core Logic Implementation Based on Dummy (Keyboard/Mouse) Input | Individual Dev |
| Step 2 | Week 4 | Housing Fabrication (3D Modeling) Improving ergonomics | UI/UX Design Calibration UI & Logic Implementation | Mid-Review |
| Step 3 | Week 5 | [Integration] HW-SW Physical Link Input Latency Optimization | [Integration] Data Parsing Logic Effect & Sound Polishing | Final Build |

5. Tech Stack

- Game Engine: Unreal Engine 5 (Blueprint & C++)

- MCU (Microcontroller): Arduino Uno or ESP32 [Owned by Younghwan Shin]

- Sensor: [MPU6050 (Gyro/Accel) / Pressure Sensor, etc.]

- Communication: Serial Communication (UART) via UE4Duino Plugin

- Collaboration: GitHub

6. Roles & Responsibilities (R&R)

- Team Leader/PM: Overall schedule management, proposal refinement, preparation of presentation materials (PPT) and demo videos.

- HW Developer: Circuit design & soldering, sensor data preprocessing code, housing (3D) fabrication.

- SW Developer (System): Unreal-Arduino serial communication integration, data calibration algorithm implementation.

- SW Developer (Content): Game level design, UI/UX creation, asset optimization.

7. Expected Outcome

- Social Impact: Reducing the burden of expensive rehabilitation costs and improving quality of life through a fun rehabilitation culture.

- Technical Impact: Securing real-time bidirectional interaction technology between Hardware (IoT) and Game Engines.

- Competition Edge: Persuading judges by emphasizing "Warm technology for the socially vulnerable" combined with a "Feasible, working prototype."